| Tien Thanh Education and Training Department Tien Thanh secondary school | GIFTED STUDENTS EXAMINATION SUBJECT: PHYSICS 9 Shool year: 2017 – 2018 |
| --- | --- |

Ex1: a 10-meter long copper conducting-wire (l1=10m) has an resistance R1 and a 5-meter long aluminium conducting-wire (l2=5m) has an resistance R2. Compare between R1 and R2.

Answer:

Ex2: a homogeneous conducting-wire, which has the length l, the section S and an 8Ω resistance, is bent half in length. Calculate the amount of the new conducting-wire’s resistance.

Answer:

Ex3: when putting a potential difference into 2 tops of a conducting-wire, we have an electric current having an amount of 6mA. If you want the amount of the current to decrease 4mA smaller, the amount of the potential difference must be…….

Answer:

Ex4: Two conductors are connected in series, one wire with length l1 = 2m, cross section S1 = 0.5mm². The other wire length is l2 = 1m, cross section S2 = 1mm². The relationship of the heat generated on each conductor is as follows:

Answer:

Ex5: we have two resistances: R1=20Ω can stand a current having a maximum intensity of 2A and R2=40Ω can stand a current having a maximum intensity of 1,5A. The maximum amount of the potential difference can be put into 2 tops of a series circuit consisting of R1 and R2 is….

Answer:

Ex6: a copper wire, which is 10 meters in length, has a section of 0,2 mm2. Copper has resistivity 1,7.108. Calculate the amount of the resistance.

Answer:

Ex7: when putting a bulb into 6V potential difference, the current running through it has the intensity of 0,2A. Calculate the consumed power of the bulb.

Answer:

Ex8: we have wires made from same materials. If their length triples and section decreases 3 times, the intensity of the wires will be…….

Answer:

Ex9: When placing a 12V voltage at the ends of a winding wire, the current through it is 1.5A. The length of the wire used to wind the coil is (Knowing that this type of conductor if 6m long has a resistance of 2 .)

Answer:

Ex10: an electrical stove having a tag written 220V-1000W is used with the amount of potential difference 220V. Calculate the amount of its resistance.

Answer:

Ex11: Bulb D1 has assessing potential difference U1=1,5V and bulb D2 has assessing potential difference U2=6V. When they are normally lighting, the amount of their resistances are R1=1,5Ω and R2=8Ω. These two bulbs are put into one rheostat and a potential difference with U=7,5 V. Basing on the graph below, how much can we change the amount of rheostat so that these two bulb light normally?

ANSWERS :

| Ex | Answers |  |
| --- | --- | --- |
| 1 | There is not enough data to compare |  |
| 2 | 2 Ω |  |
| 3 | 4V |  |
| 4 | 110 Ω; 0,2A |  |
| 5 | 90V |  |
| 6 | 0,85 Ω |  |
| 7 | 1,2W |  |
| 8 | Increasing more than 9 times |  |
| 9 | 24m |  |
| 10 | R= 48,4 Ω |  |

EX11:- To have 2 nomally light bulbs, the intensity of the electric current that runs through each bulb is the same as the electric current norm running through each bulb.

I1= = = 1(A)

I2= 0,75A

- because the circuit consists of : Đ1nt( Đ2// Rb) , we have:

Ib= I1- I2= 1- 0,75= 0,25(A)

Ub= U2= 6V                      (0,5 đ)

- when the rheostat has the resistance, it will be:

Rb= = 24( )